| № | Description | Technical specifications | Quantity |
| --- | --- | --- | --- |
|  | Line-19-2 AES SPP-A-220-060-N-UXL4 (or equivalent) | 1. For industrial premises. 2. Ingress protection rating, IP: not less than 65. 3.Ambient temperature range, °C: minimum not more than - 30, maximum not less than +60. 4. AC voltage range, V: minimum not more than 140, maximum not less than 265. 5. DC voltage range, V: minimum not more than 200, maximum not more than 370. 6. Service life, h: not less than 100000. 7. Mounting - wall, ceiling. 8. Luminous flux, lm: not less than 3700. 9. Power consumption, W: not more than 40. 10. Dimensions in mm - length not more than 1300, width not more than 150, depth not more than 110. 11. Weight, kg: not more than 3. 12. Color temperature, K: not less than 4000. 13. Light source type - LED. 14. Electrical shock protection class — II. | 5 pcs |
|  | Lighting Fixture Trassa-3A SPS-A-220-003-T (or equivalent) | 1. For industrial premises. 2. Ingress protection rating, IP: not less than 65. 3.Ambient temperature range, °C: minimum not more than - 63, maximum not less than +60. 4. AC voltage range, V: minimum not more than 140, maximum not less than 265. 5. DC voltage range, V: minimum not more than 200, maximum not more than 370. 6. Service life, h: not less than 100000. 7. Housing - anodized aluminum profile with plastic end caps. 8. Diffuser - polycarbonate. 9. Mounting - wall, ceiling. 10. Luminous flux, lm: not less than 11250. 11. Power consumption, W: not more than 91. 12. Dimensions in mm - length not more than 490, width not more than 220, depth not more than 250. 13. Weight, kg: not more than 7. 14. Color temperature, K: not less than 4000. 15. Light source type - LED. | 10 pcs |
|  | Lighting fixture PVLM P-2x40-002 (or equivalent) | 1. Electrical shock protection class – II. 2. For industrial premises. 3. Power, W: not more than 72. 4. Service life, h: not less than 72000. 5. Standard version. 6. Ingress protection rating, IP: not less than 54. 7. Luminous flux, lm: not less than 5000. 8. Dimensions, mm: Length not more than 1290, Width not more than 141, Height not more than 151. | 5 pcs |
|  | Armstrong-14-O lamp or equivalent | 1. For industrial premises. 2. Ingress protection rating, IP: not less than 40. 3.Ambient temperature range, °C: minimum not more than - 30, maximum not less than +60. 4. AC voltage range, V: minimum not more than 140, maximum not less than 265. 5. DC voltage range, V: minimum not more than 200, maximum not more than 370. 6. Service life, h: not less than 100000. 7. Housing - painted metal. 8. Diffuser - opalescent polycarbonate. 9. Mounting - wall, ceiling, recessed. 10. Luminous flux, lm: not less than 5700. 11. Power consumption, W: not more than 50. 12. Dimensions in mm - length not less than 594 and not more than 596, width not less than 594 and not more than 596, depth not more than 50. 13. Weight, kg: not more than 5. 14. Color temperature, K: not less than 4000. 15. Light source type - LED. | 55 pcs |
|  | Projector BSA 14/4-18 LED 602111850 (or equivalent) | 1. Installation method - floor - mounted. 2. Housing material - plastic. 3. Lamp type - LED 4. Luminaire power, W: not less than 25. 5. The number and voltage of the power elements, V - not less than 1x14.4V/1x18. 6. Color temperature, K — not less than 5000. 7. Luminous flux, lm: not less than 1800. 8. Ingress protection rating, IP: not less than 65. | 5 pcs |
|  | Armstrong-14-P lamp (or equivalent not less than) | 1. For industrial premises. 2. Ingress protection rating, IP: not less than 40. 3.Ambient temperature range, °C: minimum not more than - 30, maximum not less than +60. 4. AC voltage range, V: minimum not more than 140, maximum not less than 265. 5. DC voltage range, V: minimum not more than 200, maximum not more than 370. 6. Service life, h: not less than 100000. 7. Housing - painted metal. 8. Diffuser - opalescent polycarbonate. 9. Mounting - wall, ceiling, recessed. 10. Luminous flux, lm: not less than 5700. 11. Power consumption, W: not more than 50. 12. Dimensions in mm - length not less than 594 and not more than 596, width not less than 594 and not more than 596, depth not more than 50. 13. Weight, kg: not more than 5. 14. Color temperature, K: not less than 4000. 15. Light source type - LED | 10 pcs |
|  | Lighting fixture DMS-P1-042-DS4-MT (or equivalent) | 1. Ingress protection rating, IP: not less than 65. 2.Ambient temperature range, °C: minimum not more than - 40, maximum not less than +50. 3. AC voltage range, V: minimum not more than 170, maximum not less than 265. 4. Service life, h: not less than 100000. 5. Mounting - wall, ceiling. 6. Luminous flux, lm: not less than 3700. 7. Power consumption, W: not less than 40. 8. Dimensions in mm - length not more than 1300, width not more than 150, depth not more than 110. 9. Weight, kg: not more than 3. 10. Color temperature, K: not less than 4000. 11. Light source type - LED. | 30 pcs |
|  | Lighting Fixture GL-AT200A Garwin (or equivalent) | 1. For industrial premises. 2. Ingress protection rating, IP: not less than 65. 3.Ambient temperature range, °C: minimum not more than - 63, maximum not less than +60. 4. AC voltage range, V: minimum not more than 140, maximum not less than 265. 5. DC voltage range, V: minimum not more than 200, maximum not more than 370. 6. Service life, h: not less than 100000. 7. Housing - anodized aluminum profile with plastic end caps. 8. Diffuser - polycarbonate. 9. Mounting - wall, ceiling. 10. Luminous flux, lm: not less than 11250. 11. Power consumption, W: not more than 91. 12. Dimensions in mm - length not more than 490, width not more than 220, depth not more than 250. 13. Weight, kg: not more than 7. 14. Color temperature, K: not less than 4000. 15. Light source type - LED. | 20 pcs |
|  | Lighting fixture LSU-1, not less than 60W, not more than 60W, 25m (or equivalent) | 1. Housing material - plastic. 2. Cable length, m: not less than 25. 3. Socket type - E27. 4. Ingress protection rating, - not less than 20. 5. Lamp type - incandescent 6. Power, W: not more than 60. 7. Supply voltage, V: not less than 220. | 20 pcs |
|  | Lighting fixture UWL 12100-Tec (or equivalent) | 1. Plastic housing. 2. Protection class, IP: not less than 68. 3. Immersion depth, m: not less than 4. 4. Transformer protection class, IP — not less than 44. 5. Voltage, V: not more than 12, AC. 6. Dish: type base. 7. Power, W: not more than 110. 8. Includes color filters and base. 9. Size, mm — not more than 150x150. | 12 pcs |
|  | Feron LL-503 projector on a tripod (or equivalent) | 1. Spotlight on a tripod. 2. Number of spotlights - not less than 2. 3. Power, W - not more than 55 for one spotlight. 4. Mounting type - portable. 5. Color temperature, K — not less than 6400. 6. Ingress protection rating, IP: not less than 65. 7. Luminaire housing material - aluminum 8. Height, mm: not less than 1700. 9. Length, mm: not more than 700. 8. Voltage, V: not less than 230. 9. Luminous flux, lm: not less than 9000. 10. Width, mm: not less than 900. 11. Operating temperature range, °C: minimum not more than - 40, maximum not less than +40. 12. Material - aluminum, glass | 20 pcs |
|  | Portable Lamp JAZ-0070B (or equivalent) | 1. Light source - LED. 2. Type of socket - built-in LED 3. Type of power supply - from Li-ION battery 4. Ingress protection rating, IP: not less than 44. 5. Luminous flux, lm: not less than 280. 6. Charging time, h: not more than 4. 7. Dimensions, mm — not more than 400x12x150. 8. Material - plastic. 9. Built - in neodymium magnets. 10. Charger included. 11. Battery capacity, Ah — not less than 2.25. 12. Operating time, h: not less than 8. | 6 pcs |
|  | Indicator a15718 BS-73-S1-INEXI2 (or equivalent) | 1. Ingress protection rating - not less than IP65. 2. Height, mm: not more than 240. 3. Length, mm: not more than 340. 4. Width, mm: not more than 60. 5. Power, W: not more than 15. 6. Voltage, V: not less than 230. 7. Protection class - not less than II. 8. Autonomous operating time, h: not less than 3. 9. With a button simulating disconnection of the mains supply 10. Operating mode - continuous. 11. Lamp type - LED. | 155 pcs |
|  | Light fixture V1-R0-00555-10A00-6501030 (or equivalent) | 1. For industrial premises. 2. Ingress protection rating, IP: not less than 65. 3.Ambient temperature range, °C: minimum not more than - 30, maximum not less than +40. 4. AC voltage range, V: minimum not more than 220, maximum not less than 240. 5. Luminous flux, lm: not less than 900. 9. Power consumption, W: not more than 10. 10. Dimensions in mm - diameter not more than 105, depth not more than 60. 11. Housing material - aluminum. 12. Color temperature, K: not less than 3000. 13. Light source type - LED. 14. Electrical shock protection class — II. 15. Mounting hole, mm — not less than 89 and not more than 91. | 50 pcs |
|  | AES Line-19-2 (or equivalent) | 1. For industrial premises. 2. Ingress protection rating, IP: not less than 65. 3.Ambient temperature range, °C: minimum not more than - 30, maximum not less than +60. 4. AC voltage range, V: minimum not more than 140, maximum not less than 265. 5. DC voltage range, V: minimum not more than 200, maximum not more than 370. 6. Service life, h: not less than 100000. 7. Mounting - wall, ceiling. 8. Luminous flux, lm: not less than 3700. 9. Power consumption, W: not more than 40. 10. Dimensions in mm - length not more than 1300, width not more than 150, depth not more than 110. 11. Weight, kg: not more than 3. 12. Color temperature, K: not less than 4000. 13. Light source type - LED. 14. Electric shock protection class - II | 260 pcs |
|  | Projector FL-LED Light-PAD 50W (or equivalent) | 1. Type of source - LED. 2.Battery operating time, h: not less than 4. 3.Luminous flux, lm: not less than 4250. 4. Battery voltage, V - not less than 16.8. 5. Recharge time, h: not more than 3. 6. Service life, h: not less than 30000. 7. Color temperature, K - not less than 4200. 8. Dimensions in mm - length not less than 224 and not more than 226, width not less than 243 and not more than 245, height not less than 391 and not more than 393. | 21 pcs |
|  | Lighting Fixture CD LED 27 4000K 1134000020 (or equivalent) | 1. Power, W: not more than 27. 2. Light source type - LED. 3. Color temperature, K: not less than 4000. 4. Luminous flux, lm: not less than 2500. 5. Ingress protection rating, IP: not less than 65. 6. Diameter, mm: not more than 400. 7. Weight, kg: not more than 3. 8. Material - polycarbonate. 9. Service life, h: not less than 50 000. 10. The diffuser material is polymethylmethacrylate. 11. Depth, mm: not more than 150. 12. Ambient temperature range, °C: minimum not more than - 20, maximum not less than +40. 13. Rated voltage, V: not less than 230. 14. Electrical shock protection class — II. | 10 pcs |
|  | Lamp Vizir-1 SSO-A-220-046-N-HXL4 (or equivalent) | 1. For industrial premises. 2. Ingress protection rating, IP: not less than 54. 3.Ambient temperature range, °C: minimum not more than - 10, maximum not less than +60. 4. AC voltage range, V: minimum not more than 140, maximum not less than 265. 5. DC voltage range, V: minimum not more than 200, maximum not more than 370. 6. Service life, h: not less than 100000. 7. Housing - painted metal. 8. Diffuser - opalescent polycarbonate. 9. Mounting - wall, ceiling, recessed. 10. Luminous flux, lm: not less than 960. 11. Power consumption, W: not more than 15. 12. Dimensions, mm: Diameter not more than 200, Width not more than 220. 13. Weight, kg: not more than 3. 14. Color temperature, K: not less than 4000. 15. Borosilicate glass. 16. Light source type - LED. | 10 pcs |
|  | Lighting Fixture Trassa-6 SCAS-A-220-006-N-HXL1 (or equivalent) | 1. For outdoor lighting. 2. Ingress protection rating, IP: not less than 67. 3.Ambient temperature range, °C: minimum not more than - 63, maximum not less than +60. 4. AC voltage range, V: minimum not more than 140, maximum not less than 265. 5. DC voltage range, V: minimum not more than 200, maximum not more than 370. 6. Service life, h: not less than 100000. 7. Housing - anodized aluminum profile with plastic end caps. 8. Diffuser - polycarbonate. 9. Mounting - bracket. 10. Luminous flux, lm: not less than 22500. 11. Power consumption, W: not more than 190. 12. Dimensions in mm - length not more than 900, width not more than 220, depth not more than 250. 13. Weight, kg: not more than 11. 14. Color temperature, K: not less than 4000. 15. Light source type - LED. | 5 pcs |
|  | Lighting fixture SSDP Briz 30, 30W, 4050lm, IP65 (or equivalent) | 1.Power, W: not more than 31. 2. Power supply voltages, V - not more than 12 and not less than 36. 3. Supply voltage - VAC/VDC. 4.Luminous flux, lm: not less than 4050. 5. Color temperature, K - not more than 5000. 6.Dimensions, mm: Length not more than 253, Width not more than 113, Thickness not more than 75. 7. Ripple factor, % - not more than 1. 8.Service life, h: not less than 100000. 9.Ingress protection rating, IP: not less than 65. 10.Ambient temperature, °C: minimum not more than - 40, maximum not less than +40. 11.Housing material - aluminum. 12. Protective cover - polycarbonate. 13. Light source type - LED. | 5 pcs |
|  | LZ.OPL ECO LED Light Bulb, 1500 lm (not more than), 5000K (or equivalent) | 1. Ingress protection rating, IP: not less than 65. 2.Ambient temperature range, °C: minimum not more than - 63, maximum not less than +60. 4. AC voltage range, V: minimum not more than 198, maximum not less than 264. 5. DC voltage range, V: minimum not more than 198, maximum not more than 264. 6. Service life, h: not less than 100000. 7. Housing - anodized aluminum profile with plastic end caps. 8. Diffuser - polycarbonate. 9. Mounting - bracket. 10. Luminous flux, lm: not less than 7200. 11. Power consumption, W: not more than 62. 12. Dimensions in mm - length not more than 1600, width not more than 200, depth not more than 110. 13. Weight, kg: not more than 7. 14. Color temperature, K: not less than 5000. 15. Light source type - LED. 16. With emergency backup power unit. 17. Battery operating time, h: not more than 1. 18. Light output in emergency mode, lm - not less than 550. | 10 pcs |