| № | Description | Technical specifications | Quantity | Unit |
| --- | --- | --- | --- | --- |
| 1 | Outdoor luminaire | Power greater than 40 and not more than 70 W Type of luminaire construction Consoles not less than Correlated color temperature, min not less than 3000 K Correlated color temperature, max not more than 3000 K Luminous flux greater than 8000 and not more than 10000 lm Power factor not less than 0,95 First digit of IP rating not less than 6 Second digit of IP rating not less than 5 Color rendering index not less than 70 and less than 80 Electric shock protection class I Light source Solid-state lighting module not more than Ripple factor greater than 1 and not more than 5 % Type of light curve characteristic Wide Type of conditional equatorial curve of light strength Boxicidental Type of light distribution in the glare zone Limited Net weight greater than 1 and not more than 5 kg Luminaire length not less than 550 and less than 650 mm Luminaire width not less than 200 and less than 300 mm Luminaire height not less than 50 and less than 150 mm Mountings included Yes Material of the diffuser: Tempered glass Housing material aluminum Light output with color rendering index not less than 70 and less than 80, lm/W ≥ 130 IK protection rating 08 Mechanical execution group M2 Road width to be illuminated less than 7 m Requirements for Equipment and Functions: Presence of anti-corrosion coating on the luminaire housing; Serviceability without lamp removal; The possibility of installing a light fixture on a pipe; Presence of power supply cable for the luminaire; The possibility of adjusting the tilt angle not less than 0° and not more than 90°. Maximum operating temperature, degrees 40 Minimum operating temperature, degrees not less than -40 | 2073 | pcs. |
| 2 | Outdoor luminaire | Power greater than 40 and not more than 70 W Type of luminaire construction Pendant Correlated color temperature, min not less than 3000 K Correlated color temperature, max not more than 3000 K Luminous flux greater than 8000 and not more than 10000 lm Power factor not less than 0,95 First digit of IP rating not less than 6 Second digit of IP rating not less than 5 Color rendering index not less than 70 and less than 80 Electric shock protection class I Light source Solid-state lighting module not more than Ripple factor greater than 1 and not more than 5 % Type of light curve characteristic Wide Type of conditional equatorial curve of light strength Axial Type of light distribution in the glare zone Limited Net weight greater than 10 and not more than 20 kg Luminaire length not less than 350 and less than 550 mm Luminaire width not less than 300 and less than 500 mm Luminaire height not less than 150 and less than 250 mm Mountings included Yes Material of the diffuser: Tempered glass Housing material aluminum Light output with color rendering index not less than 70 and less than 80, lm/W ≥ 130 IK protection rating 08 Mechanical execution group M2 Road width to be illuminated less than 7 m Requirements for Equipment and Functions: Presence of anti-corrosion coating on the luminaire housing; Serviceability without lamp removal; Presence of power supply cable for the luminaire; The possibility of adjusting the tilt angle not less than 0° and not more than 90°. Maximum operating temperature, degrees 40 Minimum operating temperature, degrees not less than -40 | 103 | pcs. |
| 3 | Outdoor luminaire | Power greater than 70 and not more than 100 W Type of luminaire construction Consoles not less than Correlated color temperature, min not less than 3000 K Correlated color temperature, max not more than 3000 K Luminous flux greater than 10000 and not more than 15000 lm Power factor not less than 0,95 First digit of IP rating not less than 6 Second digit of IP rating not less than 5 Color rendering index not less than 70 and less than 80 Electric shock protection class I Light source Solid-state lighting module not more than Ripple factor greater than 1 and not more than 5 % Type of light curve characteristic Wide Type of conditional equatorial curve of light strength Boxicidental Type of light distribution in the glare zone Limited Net weight greater than 1 and not more than 5 kg Luminaire length not less than 550 and less than 650 mm Luminaire width not less than 200 and less than 300 mm Luminaire height not less than 50 and less than 150 mm Mountings included Yes Material of the diffuser: Tempered glass Housing material aluminum Light output with color rendering index not less than 70 and less than 80, lm/W ≥ 130 IK protection rating 08 Mechanical execution group M2 Road width to be illuminated less than 7 m Requirements for Equipment and Functions: Presence of anti-corrosion coating on the luminaire housing; Serviceability without lamp removal; The possibility of installing a light fixture on a pipe; Presence of power supply cable for the luminaire; The possibility of adjusting the tilt angle not less than 0° and not more than 90°. Maximum operating temperature, degrees 40 Minimum operating temperature, degrees not less than -40 | 2115 | pcs. |
| 4 | Outdoor luminaire | Power greater than 70 and not more than 100 W Type of luminaire construction Consoles not less than Correlated color temperature, min not less than 3000 K Correlated color temperature, max not more than 3000 K Luminous flux greater than 10000 and not more than 15000 lm Power factor not less than 0,95 First digit of IP rating not less than 6 Second digit of IP rating not less than 5 Color rendering index not less than 70 and less than 80 Electric shock protection class I Light source Solid-state lighting module not more than Ripple factor greater than 1 and not more than 5 % Type of light curve characteristic Wide Type of conditional equatorial curve of light strength Boxicidental Type of light distribution in the glare zone Limited Net weight greater than 1 and not more than 5 kg Luminaire length not less than 550 and less than 650 mm Luminaire width not less than 200 and less than 300 mm Luminaire height not less than 50 and less than 150 mm Mountings included Yes Material of the diffuser: Tempered glass Housing material aluminum Light output with color rendering index not less than 70 and less than 80, lm/W ≥ 130 IK protection rating 08 Mechanical execution group M2 Road width to be illuminated not less than 7 and less than 16 m Requirements for Equipment and Functions: Presence of anti-corrosion coating on the luminaire housing; Serviceability without lamp removal; The possibility of installing a light fixture on a pipe; Presence of power supply cable for the luminaire; The possibility of adjusting the tilt angle not less than 0° and not more than 90°. Maximum operating temperature, degrees 40 Minimum operating temperature, degrees not less than -40 | 319 | pcs. |
| 5 | Outdoor luminaire | Power greater than 70 and not more than 100 W Type of luminaire construction Consoles not less than Correlated color temperature, min not less than 5000 K Correlated color temperature, max not more than 5000 K Luminous flux greater than 10000 and not more than 15000 lm Power factor not less than 0,95 First digit of IP rating not less than 6 Second digit of IP rating not less than 5 Color rendering index not less than 70 and less than 80 Electric shock protection class I Light source Solid-state lighting module not more than Ripple factor greater than 1 and not more than 5 % Type of light curve characteristic Wide Type of conditional equatorial curve of light strength Boxicidental Type of light distribution in the glare zone Limited Net weight greater than 1 and not more than 5 kg Luminaire length not less than 550 and less than 650 mm Luminaire width not less than 200 and less than 300 mm Luminaire height not less than 50 and less than 150 mm Mountings included Yes Material of the diffuser: Tempered glass Housing material aluminum Light output with color rendering index not less than 70 and less than 80, lm/W ≥ 130 IK protection rating 08 Mechanical execution group M2 Road width to be illuminated not less than 7 and less than 16 m Requirements for Equipment and Functions: Presence of anti-corrosion coating on the luminaire housing; Serviceability without lamp removal; The possibility of installing a light fixture on a pipe; Presence of power supply cable for the luminaire; The possibility of adjusting the tilt angle not less than 0° and not more than 90°. Maximum operating temperature, degrees 40 Minimum operating temperature, degrees not less than -40 | 15 | pcs. |
| 6 | Outdoor luminaire | Power greater than 70 and not more than 100 W Type of luminaire construction Pendant Correlated color temperature, min not less than 3000 K Correlated color temperature, max not more than 3000 K Luminous flux greater than 10000 and not more than 15000 lm Power factor not less than 0,95 First digit of IP rating not less than 6 Second digit of IP rating not less than 5 Color rendering index not less than 70 and less than 80 Electric shock protection class I Light source Solid-state lighting module not more than Ripple factor greater than 1 and not more than 5 % Type of light curve characteristic Wide Type of conditional equatorial curve of light strength Axial Type of light distribution in the glare zone Limited Net weight greater than 10 and not more than 20 kg Luminaire length not less than 350 and less than 550 mm Luminaire width not less than 300 and less than 500 mm Luminaire height not less than 150 and less than 250 mm Mountings included Yes Material of the diffuser: Tempered glass Housing material aluminum Light output with color rendering index not less than 70 and less than 80, lm/W ≥ 130 IK protection rating 08 Mechanical execution group M2 Road width to be illuminated less than 7 m Requirements for Equipment and Functions: Presence of anti-corrosion coating on the luminaire housing; Serviceability without lamp removal; Presence of power supply cable for the luminaire; The possibility of adjusting the tilt angle not less than 0° and not more than 90°. Maximum operating temperature, degrees 40 Minimum operating temperature, degrees not less than -40 | 423 | pcs. |
| 7 | Outdoor luminaire | Power greater than 70 and not more than 100 W Type of luminaire construction Pendant Correlated color temperature, min not less than 5000 K Correlated color temperature, max not more than 5000 K Luminous flux greater than 10000 and not more than 15000 lm Power factor not less than 0,95 First digit of IP rating not less than 6 Second digit of IP rating not less than 5 Color rendering index not less than 70 and less than 80 Electric shock protection class I Light source Solid-state lighting module not more than Ripple factor greater than 1 and not more than 5 % Type of light curve characteristic Wide Type of conditional equatorial curve of light strength Axial Type of light distribution in the glare zone Limited Net weight greater than 10 and not more than 20 kg Luminaire length not less than 350 and less than 550 mm Luminaire width not less than 300 and less than 500 mm Luminaire height not less than 150 and less than 250 mm Mountings included Yes Material of the diffuser: Tempered glass Housing material aluminum Light output with color rendering index not less than 70 and less than 80, lm/W ≥ 130 IK protection rating 08 Mechanical execution group M2 Road width to be illuminated less than 7 m Requirements for Equipment and Functions: Presence of anti-corrosion coating on the luminaire housing; Serviceability without lamp removal; Presence of power supply cable for the luminaire; The possibility of adjusting the tilt angle not less than 0° and not more than 90°. Maximum operating temperature, degrees 40 Minimum operating temperature, degrees not less than -40 | 49 | pcs. |
| 8 | Outdoor luminaire | Power greater than 100 and not more than 130 W Type of luminaire construction Consoles not less than Correlated color temperature, min not less than 3000 K Correlated color temperature, max not more than 3000 K Luminous flux greater than 15000 and not more than 20000 lm Power factor not less than 0,95 First digit of IP rating not less than 6 Second digit of IP rating not less than 5 Color rendering index not less than 70 and less than 80 Electric shock protection class I Light source Solid-state lighting module not more than Ripple factor greater than 1 and not more than 5 % Type of light curve characteristic Wide Type of conditional equatorial curve of light strength Boxicidental Type of light distribution in the glare zone Limited Net weight greater than 5 and not more than 10 kg Luminaire length not less than 650 and less than 1000 mm Luminaire width not less than 300 and less than 500 mm Luminaire height not less than 50 and less than 150 mm Mountings included Yes Material of the diffuser: Tempered glass Housing material aluminum Light output with color rendering index not less than 70 and less than 80, lm/W ≥ 130 IK protection rating 08 Mechanical execution group M2 Road width to be illuminated less than 7 m Requirements for Equipment and Functions: Presence of anti-corrosion coating on the luminaire housing; Serviceability without lamp removal; The possibility of installing a light fixture on a pipe; Presence of power supply cable for the luminaire; The possibility of adjusting the tilt angle not less than 0° and not more than 90°. Maximum operating temperature, degrees 40 Minimum operating temperature, degrees not less than -40 | 381 | pcs. |
| 9 | Outdoor luminaire | Power greater than 100 and not more than 130 W Type of luminaire construction Consoles not less than Correlated color temperature, min not less than 5000 K Correlated color temperature, max not more than 5000 K Luminous flux greater than 15000 and not more than 20000 lm Power factor not less than 0,95 First digit of IP rating not less than 6 Second digit of IP rating not less than 5 Color rendering index not less than 70 and less than 80 Electric shock protection class I Light source Solid-state lighting module not more than Ripple factor greater than 1 and not more than 5 % Type of light curve characteristic Wide Type of conditional equatorial curve of light strength Boxicidental Type of light distribution in the glare zone Limited Net weight greater than 5 and not more than 10 kg Luminaire length not less than 650 and less than 1000 mm Luminaire width not less than 300 and less than 500 mm Luminaire height not less than 50 and less than 150 mm Mountings included Yes Material of the diffuser: Tempered glass Housing material aluminum Light output with color rendering index not less than 70 and less than 80, lm/W ≥ 130 IK protection rating 08 Mechanical execution group M2 Road width to be illuminated less than 7 m Requirements for Equipment and Functions: Presence of anti-corrosion coating on the luminaire housing; Serviceability without lamp removal; The possibility of installing a light fixture on a pipe; Presence of power supply cable for the luminaire; The possibility of adjusting the tilt angle not less than 0° and not more than 90°. Maximum operating temperature, degrees 40 Minimum operating temperature, degrees not less than -40 | 21 | pcs. |
| 10 | Outdoor luminaire | Power greater than 100 and not more than 130 W Type of luminaire construction Consoles not less than Correlated color temperature, min not less than 3000 K Correlated color temperature, max not more than 3000 K Luminous flux greater than 15000 and not more than 20000 lm Power factor not less than 0,95 First digit of IP rating not less than 6 Second digit of IP rating not less than 5 Color rendering index not less than 70 and less than 80 Electric shock protection class I Light source Solid-state lighting module not more than Ripple factor greater than 1 and not more than 5 % Type of light curve characteristic Wide Type of conditional equatorial curve of light strength Boxicidental Type of light distribution in the glare zone Limited Net weight greater than 5 and not more than 10 kg Luminaire length not less than 650 and less than 1000 mm Luminaire width not less than 300 and less than 500 mm Luminaire height not less than 50 and less than 150 mm Mountings included Yes Material of the diffuser: Tempered glass Housing material aluminum Light output with color rendering index not less than 70 and less than 80, lm/W ≥ 130 IK protection rating 08 Mechanical execution group M2 Road width to be illuminated not less than 7 and less than 16 m Requirements for Equipment and Functions: Presence of anti-corrosion coating on the luminaire housing; Serviceability without lamp removal; The possibility of installing a light fixture on a pipe; Presence of power supply cable for the luminaire; The possibility of adjusting the tilt angle not less than 0° and not more than 90°. Maximum operating temperature, degrees 40 Minimum operating temperature, degrees not less than -40 | 429 | pcs. |
| 11 | Outdoor luminaire | Power greater than 130 and not more than 160 W Type of luminaire construction Consoles not less than Correlated color temperature, min not less than 5000 K Correlated color temperature, max not more than 5000 K Luminous flux greater than 20000 and not more than 25000 lm Power factor not less than 0,95 First digit of IP rating not less than 6 Second digit of IP rating not less than 5 Color rendering index not less than 70 and less than 80 Electric shock protection class I Light source Solid-state lighting module not more than Ripple factor greater than 1 and not more than 5 % Type of light curve characteristic Wide Type of conditional equatorial curve of light strength Boxicidental Type of light distribution in the glare zone Limited Net weight greater than 5 and not more than 10 kg Luminaire length not less than 650 and less than 1000 mm Luminaire width not less than 300 and less than 500 mm Luminaire height not less than 50 and less than 150 mm Mountings included Yes Material of the diffuser: Tempered glass Housing material aluminum Light output with color rendering index not less than 70 and less than 80, lm/W ≥ 130 IK protection rating 08 Mechanical execution group M2 Road width to be illuminated less than 7 m Requirements for Equipment and Functions: Presence of anti-corrosion coating on the luminaire housing; Serviceability without lamp removal; The possibility of installing a light fixture on a pipe; Presence of power supply cable for the luminaire; The possibility of adjusting the tilt angle not less than 0° and not more than 90°. Maximum operating temperature, degrees 40 Minimum operating temperature, degrees not less than -40 | 116 | pcs. |
| 12 | Outdoor luminaire | Power greater than 130 and not more than 160 W Type of luminaire construction Consoles not less than Correlated color temperature, min not less than 3000 K Correlated color temperature, max not more than 3000 K Luminous flux greater than 20000 and not more than 25000 lm Power factor not less than 0,95 First digit of IP rating not less than 6 Second digit of IP rating not less than 5 Color rendering index not less than 70 and less than 80 Electric shock protection class I Light source Solid-state lighting module not more than Ripple factor greater than 1 and not more than 5 % Type of light curve characteristic Wide Type of conditional equatorial curve of light strength Boxicidental Type of light distribution in the glare zone Limited Net weight greater than 5 and not more than 10 kg Luminaire length not less than 650 and less than 1000 mm Luminaire width not less than 300 and less than 500 mm Luminaire height not less than 50 and less than 150 mm Mountings included Yes Material of the diffuser: Tempered glass Housing material aluminum Light output with color rendering index not less than 70 and less than 80, lm/W ≥ 130 IK protection rating 08 Mechanical execution group M2 Road width to be illuminated not less than 7 and less than 16 m Requirements for Equipment and Functions: Presence of anti-corrosion coating on the luminaire housing; Serviceability without lamp removal; The possibility of installing a light fixture on a pipe; Presence of power supply cable for the luminaire; The possibility of adjusting the tilt angle not less than 0° and not more than 90°. Maximum operating temperature, degrees 40 Minimum operating temperature, degrees not less than -40 | 1266 | pcs. |
| 13 | Outdoor luminaire | Power greater than 130 and not more than 160 W Type of luminaire construction Consoles not less than Correlated color temperature, min not less than 5000 K Correlated color temperature, max not more than 5000 K Luminous flux greater than 20000 and not more than 25000 lm Power factor not less than 0,95 First digit of IP rating not less than 6 Second digit of IP rating not less than 5 Color rendering index not less than 70 and less than 80 Electric shock protection class I Light source Solid-state lighting module not more than Ripple factor greater than 1 and not more than 5 % Type of light curve characteristic Wide Type of conditional equatorial curve of light strength Boxicidental Type of light distribution in the glare zone Limited Net weight greater than 5 and not more than 10 kg Luminaire length not less than 650 and less than 1000 mm Luminaire width not less than 300 and less than 500 mm Luminaire height not less than 50 and less than 150 mm Mountings included Yes Material of the diffuser: Tempered glass Housing material aluminum Light output with color rendering index not less than 70 and less than 80, lm/W ≥ 130 IK protection rating 08 Mechanical execution group M2 Road width to be illuminated not less than 7 and less than 16 m Requirements for Equipment and Functions: Presence of anti-corrosion coating on the luminaire housing; Serviceability without lamp removal; The possibility of installing a light fixture on a pipe; Presence of power supply cable for the luminaire; The possibility of adjusting the tilt angle not less than 0° and not more than 90°. Maximum operating temperature, degrees 40 Minimum operating temperature, degrees not less than -40 | 181 | pcs. |