| № | Product name | Functional characteristics (consumer properties) and quality characteristics of the product | Quantity | Unit change not less than 10% and not more than 20% |
| --- | --- | --- | --- | --- |
| 1 | Lead - acid battery for starting piston engines | OKPD 27.20.21.000 CGWS 27.20.21.000 - 00000011 Maximum battery capacity not more than 30.0 Ah (3,6 kC) Minimum battery capacity not less than 20.0 Ah (3,6 kC) Battery polarity - Reverse Battery starting current - greater than 300.0 and not more than 400.0 A Battery type - Electrolyte immobilized in the pores of a separator made of glass mat (AGM) Voltage 12 V Maintenance - maintenance - free Additional characteristics: * Terminals - motorcycle bolt terminals | 2 | pcs. |
| 2 | Lead - acid battery for starting piston engines 6 ST-12 not less than 12 V, not more than 15 V | OKPD 27.20.21.000 CGWS 27.20.21.000 - 00000001 Maximum battery capacity not more than 20.0 Ah (3,6 kC) Minimum battery capacity not less than 10.0 Ah (3,6 kC) Battery polarity - Direct Battery starting current greater than 200.0 and not more than 300.0 A Battery type - Electrolyte immobilized in the pores of a separator made of glass mat (AGM) Voltage 12 V Maintenance - maintenance - free Additional characteristics: * Type 6 ST-12 not less than 12 V, not more than 15 V | 1 | pcs. |
| 3 | Lead - acid battery for starting piston engines 6 ST-60 not less than 60 W not more than 65 W | OKPD 27.20.21.000 CGWS 27.20.21.000 - 00000001 Maximum battery capacity not more than 70.0 Ah (3,6 kC) Minimum battery capacity not less than 60.0 Ah (3,6 kC) Battery polarity - Direct Battery starting current greater than 600.0 and not more than 700.0 A Battery type - open - type batteries Voltage 12 V Maintenance - serviceable Additional characteristics: * Type 6 ST-60 not less than 60 W not more than 65 W | 12 | pcs. |
| 4 | Lead - acid battery for starting piston engines 6 ST-75 not less than 75 | OKPD 27.20.21.000 CGWS 27.20.21.000 - 00000001 Maximum battery capacity not more than 80.0 Ah (3,6 kC) Minimum battery capacity not less than 70.0 Ah (3,6 kC) Battery polarity - Direct Battery starting current - greater than 700.0 and not more than 800.0 A Battery type - open - type batteries Voltage 12 V Maintenance - serviceable Additional characteristics: * Type 6 ST-75 not less than 75 | 12 | pcs. |
| 5 | Lead - acid battery for starting piston engines 6 ST-90 not less than 90 | OKPD 27.20.21.000 CGWS 27.20.21.000 - 00000001 Maximum battery capacity not more than 100.0 Ah (3,6 kC) Minimum battery capacity not less than 90.0 Ah (3,6 kC) Battery polarity - Direct Battery starting current - greater than 700.0 and not more than 800.0 A Battery type - open - type batteries Voltage 12 V Maintenance - serviceable Additional characteristics: * Type 6 ST-90 not less than 90 | 34 | pcs. |
| 6 | Lead - acid battery for starting piston engines 6 ST-190 | OKPD 27.20.21.000 CGWS 27.20.21.000 - 00000001 Maximum battery capacity not more than 200.0 Ah (3,6 kC) Minimum battery capacity not less than 190.0 Ah (3,6 kC) Battery polarity - Direct Battery starting current - greater than 1200.0 A Battery type - open - type batteries Maintenance - serviceable Additional characteristics: * Type 6 ST-190 | 41 | pcs |
| 7 | Lead - acid battery for starting piston engines 6 ST-225 not less than 225 mm not more than | OKPD 27.20.21.000 CGWS 27.20.21.000 - 00000011 Maximum battery capacity not more than 240.0 Ah (3,6 kC) Minimum battery capacity not less than 220.0 Ah (3,6 kC) Battery polarity - Reverse Battery starting current - greater than 1200.0 A Battery type - open - type batteries Maintenance - serviceable Additional characteristics: * Type 6 ST-225 not less than 225 mm not more than | 2 | pcs. |